Simplify the IPv6 address:

2001:0718:0002:0001:1000:92e0:814a:8336

2001:0018:0000:10a0:0000:0000:215a:3548

2001:0000:0000:0000:92e0:0000:0000:0bcd

2002:0124:0015:0001:abd5:5510:0000:abba

Create EUI-64 from the following MAC addresses:

50:E5:49:E1:38:F8

00:0C:29:DA:A4:D5

00:0A:AB:54:59:53

30:d7:aa:bb:cc:dd

Mark the network area corresponding to the description.

| INTERNET INTERNET - local loop | - local link address |
| --- | --- |
| INTERNET INTERNET - global individual address | - local address |

 – symbol of an interface

Sort the IP datagram properties in the list below.

| - IPv4 | - IPv6 |
| --- | --- |

- 1 – Address length is 32 bits

- 2 – Address length is 128 bits

- 3 – Header contains checksum

- 4 – Header does not contain checksum

- 5 – Optional header items are contained in standard packet header

- 6 – Optional header items are contained in extended packet header

- 7 – Header contains bit alignment

- 8 – By default handles quality of service (QoS)

Fill in correct sequence and header names in an IPv6 datagram.

(BASE)

IPv6 Header

- If you need to transmit the header:

| - header | - type |
| --- | --- |
| - Mobility | - 135 |
| - Fragment | - 28 |
| - TCP | - 6 |
| - Authentication | - 51 |
| - Routing | - 43 |

Fill in which address may be listed in an IPv6 datagram as a source address.

- a) Computer has a single network interface that has been assigned the following addresses:

| - 1 | - local link | - fe80::20c:29ff:feda:a4d5/64 |
| --- | --- | --- |
| - 2 | - global individual | - 2001:158:12:6658:adb4:a54d:faaa:1983 |
| - 3 | - global individual | - 2001::adb4:a54d:faaa:1983 |

- Address  ___  will be listed in IPv6 datagram as a source address.

- b) Computer has a single network interface that has been assigned the following addresses:

| - 1 | - local link | - fe80::20c:29ff:fedb:a4df/64 |
| --- | --- | --- |
| - 2 | - global individual | - 2001:d844:1:1:abba:acdc:a:54 |
| - 3 | - global individual | - 2001::abba:acdc:a:54 |

- Address  ___  will be listed in IPv6 datagram as a source address.